the

Kaleidoscope

of

Marou

A living artwork.

A renewable resource.

A vision of sustainable island futures.

Normal Kaleidoscope Operation- Butterfly Mode

1. Concept Narrative

1.1 The Brief:

Building a Destination Art installation for a very small community is a challenge. It must supply electricity and drinking water while withstanding 250 km/hr.+ winds. Compounded by little or no infrastructure and minimal logistics is a difficult proposition at best. A Destination Art installation requires a certain grandeur, a je ne sais quoi, an air of the exceptional, the unexpected, amazement.

Presenting:

the Kaleidoscope of Marou

1.2 From the Brief to Inspiration: Kaleido Trays.

The Kaleidoscope of Marou is a Destination Art installation engineered for functionality, resilience, and cultural expression. Drawing inspiration from Clara von Zweigbergk’s "Kaleido" trays, the design translates modular flexibility into a system of scalable hexagonal “Modules.” Each Module is capable of generating solar energy, harvesting rainwater, water storage. Combining with other Modules to provide community space and shelter, all within a structurally resilient, symbiotic form.

The vibrant Modules can be arranged to form symbolic patterns, geometric art, or culturally significant icons, transforming a rugged infrastructure project into a living artwork co-created by the residents of Marou.

1.3 From Inspiration to Design Concept.

Scaling up from Kaleido Trays to Kaleidoscope “Modules” and adding functionalities:

The Hay “Kaleido” trays all have a rib of 11 centimeters (or multiples). Multiplying this measure with 32 gives a rib of 3,5 meters. Just like the trays, they are endlessly modular and scalable. There are 5 sizes: XS, S, M, L and XL.

This translates to the following sizes (in meters) for each Module:

| Module | Dimensions- m | Water catchment.- sqm | PV Panels- nr. | PV Panel-sqm | PV coverage- % |
| --- | --- | --- | --- | --- | --- |
| XS | 6 x 3.5 | 12,3 | 0 | 0 | 0% |
| S | 7 x 6 | 31,8 | 8 | 18,4 | 58% |
| M | 10.5 x 6 | 56,3 | 16 | 36,8 | 65% |
| L | 12.25 x 10.5 | 93,1 | 24 | 55,2 | 59% |
| XL | 14 x 12.25 | 137,1 | 36 | 82,8 | 60% |

The shapes of the Modules:

The functions are “stacked” as shown in the drawing on the right: in addition to carrying the PV Panels the Kaleidoscope Modules have an oversized “inverted umbrella” underneath the PV Panels, which captures the rainwater runoff, but also from the area surrounding the PV Panels. (the red hexagon above).

In this way land use is minimized.

The collected water is drained by a reinforced hose (here shown in blue) to the pillow tanks below

The “L” and “XL” Modules are built up from smaller Modules and are used in the lowest terraces, fixed to the ground. (the L/XL Modules have multiples of the 3.5-meter rib).

Modules include vertical mesh tarpaulin “skirts” reducing wind resistance and creating colorful walls that obscure internal systems. Ballast lines from the pillow tanks provide weight to the Modules/installation.

Right: the “S” Module in

perspective. The “inverted

umbrella” underneath

the PV Panels is visible.

1.4 From Design Concept to Design Process:

Modularity is the basis of the Kaleidoscope. By combining the modules in different ways, endless combinations of the Kaleidoscope can be formed, including the colors!

The Villagers can experiment with the physical Kaleido trays, as a model for the installation. (90 trays available). Experimenting, discussing, co-creating. Putting trays on chairs, benches and tables. Deciding on functions and colors of the trays/Modules, which sizes, which heights and location on the Design Site.

Modules can also have functions like walking paths, empty Modules for recreation, coffee corner, garden, a maze (which still harvest water) etc. by placing function “blocks” on the trays/Modules. The villagers decide on 90% of the choices! (within technical limits)

In this way terraces are shaped, as the Modules face the North in a stepped fashion to avoid shading of panels and prevent a 2D only view!

The outcome is an innovative, integrated, terraced electricity and water production site. It has a distinctive shape, which can be a place for recreation, shelter, shade and education. The height of the modules can be up to 10 meters, a great vantage point to view the entire Kaleidoscopic area.

As a contrast: a maxed

out 6 “XL” Module, pure

energy and water harvesting.

No Destination Art.
1.5 From Design Process to a Destination Art Kaleidoscope:

The following chapter is provided to demonstrate the versatility of the Kaleidoscope concept.           The Villagers will be deeply involved in the decision-making process!

“The visitor will receive a direct impression when viewing the Kaleidoscope from the south side. A large, colorful installation, standing out in its surroundings.

The visitor enters the Kaleidoscope from the South. Then walks over hexagonal tiles into a high atrium illuminated by the sun through transparent Modules in the “ceiling”. Here the fabric “skirts” become visible, that provide structure to the Modules that lie above.

The Impact Protection Barrier is used to display the cultural art of Naviti and Fiji. Also, a vertical garden can be envisaged, adding photovoltaics to the Kaleidoscope! Notice the light blue vertical pipes- Rain Harvesting!

The visitor then takes the stairs to arrive at the center terrace, in the open air, surrounded by Modules Here the South and North walk ways meet at the small green hexagon.

The terraced structure of the Kaleidoscope then becomes visible: recessed walk ways, stairs and small hideaways. Great vistas once the highest Modules are reached.

This detail shows the walkways of the Kaleidoscope. Here the visitors can enjoy the views and look for a coffee or Kava. A place for school classes on Nature, preservation and new technologies. A meeting place for the Eco Ambassadors of the Yasawa Islands!

The Kaleidoscope inner workings. As seen from the Eastern side. The pillow tanks are partly dug in.

The Kaleidoscope Overall Lay Out and Properties (example lay-out):

Energy:

200 PV Panels @ 500 Wp

North Entrance

Pick & Mix:

Fully modular

Choice of module type/size

Choice of capacities

Choice of colors

Choice of heights

Choice of functions

Choice of location

Choice of clusters

100 kWp installed

440 kWh/day av.

250 kWh battery

Water:

1000 sqm catchment

1500 cbm collection/y.

1000 cbm storage

South Entrance

1.6 From a Destination Art Kaleidoscope to Kaleidoscope Metamorphosis:

from Butterfly to Tardigrade!

Each year there is at least 1 storm stronger than force 10. The devastation of Hurricane Winston is still fresh in minds. The Kaleidoscope needs to be able to cope with 250+ km/h winds. A 48-hr. notice is given by the Fijian weather watch.

In order to avoid ultra-strong, compact, complex and expensive constructions,

the Kaleidoscope of Marou can physically adapt its structure to extreme weather conditions.

When a heavy storm warning is received, the Kaleidoscope of Marou “morphs” from an 8/10-meter-high open structure to a compact, closed structure of 2.5 meters high.

This is achieved by disconnecting the electrical connectors and the pipe couplings that connect the module to the Truss beams.   Step 1

Then sliding the whole PV Panel carrying Modules down along the vertical Truss beams. This movement is controlled by remote controlled E- winches. Step 2

Once fully lowered, the Module is reconnected to the Truss beams, same for the electrical connections, so power generation can resume. Water harvesting is not interrupted. Step 3

The Tardigrade or Water Bear is the toughest animal in the world. They can survive extreme cold, heat, vacuum, high pressures, shocks and radiation.

The outer perimeter of the Kaleidoscope is protected by a 3-meter-high Impact Protection Barrier of secured planks, reducing the wind flow under the PV Panels and protecting against the impact of 250 km/h debris.

More details on the Kaleidoscope’s hurricane passive countermeasures can be found in Par. 2.6

1.7 Community Leadership, Empowerment and Social fit:

1.7.1 Location of the Kaleidoscope on the Design Site:

The location of the Kaleidoscope will be decided by the Villagers themselves. Technical limitations will apply, like nr. and orientation of PV Panels, Panel shading avoidance, watershed flow management, building codes, location of battery room etc. Also see Par. 1.4 “Design Process”

1.7.2 Community experience & contribution:

The community experience will be a sense of achievement and pride. Knowing that most of the design, color schemes, terrace structures was developed by them. Co-creating from day One! The reliable electricity and water supply that will be provided by the Kaleidoscope will give the villagers peace of mind. The possibility for further development and ambition, as power and water are no longer a limitation. Improving daily life

1.7.3 Societal fit:

Given the conditions in Marou: limited population, skills, infrastructure, logistics etc., only a basic installation will fulfill the needs of the villagers: power and water. Technology demonstrators are interesting, but are in the given environment destined to fail. By combining the basic needs with the Kaleidoscope concept, the Destination Art objective can be achieved simultaneously.

A softer, more sensitive to the environment and surrounding nature can be achieved by incorporating “hanging gardens”. From the side of the Modules to the walk ways between the Modules, garden Modules with herbs, orchids, agrivoltaics etc. A low profile, less colorful Kaleidoscope is just as feasible & functional as the “all bells and whistles” variety!

1.7.4 Replicability & scalability:

The variety in colors, shapes, heights, empty Module functions etc. allows other Fijian and world communities to create their own Destination Art Kaleidoscope. A Kaleidoscope for other communities is a straight forward pick & mix! (and still be unique) A global constellation of Kaleidoscopic Destination Art installations can become a travel goal by itself.

1.7.5 Regionally shared resource:

The Kaleidoscope will provide ample electricity and water. Enough to share with other localities, like Yasawa School and Malevu. With the already installed PV capacity of Marou remaining online, this is a realistic scenario.

1.7.6 Education, shade & shelter:

The Kaleidoscope can be a great place for education, as a place for school classes, but also as a renewable resource demonstrator. Participating in the daily operation and maintenance of the Kaleidoscope will be valuable hands-on experience, creating young Ambassadors for renewable resources! Modules can be dedicated to this purpose.

On the South side of the Kaleidoscope and in the recessed Modules there will be shaded areas. Spaces within the Kaleidoscope can be opened up to the villagers.

Integrating the function as a shelter in the Kaleidoscope is certainly feasible, but will require engineering to higher standards.

1.7.7 Accessibility:

While not detailed in the plans, wheelchair accessibility can be provided without any major adaptations

1.7.8 Contingency:

In the worst-case scenario, (or by design) the Modules are moved to the lowest position permanently. When the excess height of the Truss beams is removed, the Kaleidoscope continues to function as a “normal” electricity and water harvest site, at full nameplate capacity. See par. 2.6

# 1.7.9 Conclusion: A resilient, beautiful and functional future!

# The Kaleidoscope of Marou transcends its basic functions. It embodies resilience, community authorship, and beauty. Offering not just energy and water but flourishing in the face of environmental and infrastructural challenges.

the

Kaleidoscope

of

Marou:

Grace in Calm,

Strength in Storm.

2.Technical Narrative

2.1 Mechanical:

For the horizontal Module construction of the hexagon Truss frames and (locally sourced) planks used. The PV Panels are mounted on the hexagonal frame. Rugged, durable, stable. Connections are made with Quick Release couplings, 750 kg x 6 / Module. Weight of a “S” Module is approx. 350 kilo total.

For the vertical structure also “Truss” type triangular beams are specified, but of a heavier specification. This allows for easy integration and interconnections between the horizontal- Modules and the -vertical- Truss beams. Size between main pipes is 50 cm.

The water capture is done with food grade HD canvas/tarp UV and fire proof, available in RAL colors.

The vertical tarpaulin skirts are perforated in order to reduce wind drag, as there is ample wind all year round on Naviti.

2.2 Electricity - PV Panels:

Any compliant glass/glass type (above 220 Wp/sqm). Glass/Glass, Mono Crystalline, Bifacial type is specified for corrosion resistance of the back panel. Orientation is East-West! The reason use E/W lay out of the Panels is that while there is a lower production peak, there is a wider peak, allowing longer capture of sunlight.

Comparison of East-West PV Panel power curve vs. pure South (or North) orientation

200 PV Panels (453 sqm) @ 500Wp/Panel gives 100kWp installed capacity. With the given average daily production of 4,4 kWh/kWp would result 440 kWh total production/day.

The East-West lay out will cost 8-10% efficiency, which can be compensated by the Bifacial set up +16-18% see below, or selecting larger, 645 Wp PV Panels for the inner mounted 4 PV Panels on a “S” Module. (2350 mm vs 2000 mm long). New PV Panels (Mar 2025) now at 23,9% efficiency.

Solar Reflector built in: The “umbrellas” under the PV Panels can be made of reflecting white tarpaulin. The PV Panel sqm is about 55% of the Module sqm. The sun’s rays also reach the umbrellas surface and reflect the light to the backside of the PV Panel. The back side of the PV Panel can also generate electricity (Bifacial).

Mounting of the PV Panels will be according to the recommendations by RMI/SEIAPI. The Modules will provide 8 bolt locations per PV Panel, and will have full support on the long side of the frame.

2.2.1 BIPV (variation on a theme)

A variation on BIPV is the possibility to use white reflection skirts of the next higher terrace, reflecting sunlight from the skirt “wall” back to the PV Panel below, enhancing production. Including from the angled surfaces from other, higher placed, Modules.

2.2.2 PV Panel Cooling

With the PV Panels optimized for maximum performance in an already warm and sunny location, overheating becomes a problem. The soft water from the Kaleidoscope can cool the Panels down with minimum effort. Low temperature coefficient PV Panels will be selected as standard.

2.3 Electricity – Optimizers and Inverters:

Each PV Panel has its own Optimizer. This Optimizer maximizes the output for each PV Panel. This allows for mix & match of PV Panels, sizes, types, orientation, shading etc. Damaged/short circuited PV Panels are automatically taken off-line, without affecting the other PV Panels performance.

The Optimizers add 2% to the PV Panels’ performance and are connected to the inverters which feed into the grid/battery.

2.4 Electricity - Battery storage:

2.6 Structural Hurricane Survival Assets of the Kaleidoscope.

The Tardigrade mode, earlier described in Par. 1.6 is the dynamic adaptation to the Hurricane threat. The following passive defensive measures have been designed into the Kaleidoscope and are always “active”.

Top Braces: the Vertical Truss beams all have Top Braces, that provide support between the Truss beams around a Module, (the green circles ) but also to the Truss beams of the adjoining Modules. This applies for both the “Butterfly” and “Tardigrade” modes.

Diagonal cross bracing: can be applied to all outer peri meter Vertical Truss beams if required. Blue circles  Additionally, Guy lines and ground anchors can be installed for further resilience (not shown)

Vertical Truss to Module cross bracing: the Kaleidoscope transfers forces through the entire installation. (the yellow circles ) The Modules are tightly connected to the Vertical Truss beams, which in turn are connected to the next Modules.

Ballast water: when the pillow tanks are 50% filled, they provide a 500 mt ballast for the Kaleidoscope. The tanks are tied to the vertical Truss beams, (the red circles ) and keep the Kaleidoscope firmly to the ground.

Impact Protection Barrier: a 3-meter-high wooden wall that surrounds the entire Kaleidoscope to protect it against flying debris and overturning winds. The black circles

Vertical Triangular Truss Beams carry the loads of the Kaleidoscope in all 3 dimensions. ()

Note: The “L” and “XL” modules are fixed at 3 meter high, and get a 0.5-meter additional wall height. (the orange triangle ), to protect the PV Panels of these Modules against debris.

Below: The Kaleidoscope in Tardigrade mode. Minimal frontage to the Hurricanes. Maximum protection. The optional cross bracing is not shown, some colors added for clarity.

with varying heights. Each module takes about 5-10 minutes and are brought down with RC controlled electrical winches. (each winch can serve 3 Modules) The couplings between the Module are of the Quick Release type, requiring minimal efforts for release and relocking. For 15 Modules about 3 hrs. with 3 people are required.

5.Environmental Impact Assessment

5.1 Land Use & Habitat Disruption

Clearing land for PV panels could impact local vegetation and wildlife. Soil erosion may increase if vegetation is removed.

5.2 Material & Manufacturing Impact

Solar panels and batteries require mining of materials like lithium, silicon, and rare earth metals.    Manufacturing and transportation of PV panels and rainwater tanks have embedded carbon emissions.

5.3 Waste Management:

End-of-life disposal of solar panels and batteries can be challenging. Rainwater harvesting tanks and pipes require maintenance and eventual replacement.